Template App TRASSIR Windows HTTP SDK

Technical Documentation

v4.1.0 | Zabbix export 7.2

| Template name | Template App TRASSIR Windows HTTP SDK |
| --- | --- |
| Purpose | Monitoring TRASSIR Server on Windows through the HTTP SDK |
| Zabbix export | 7.2 |
| Template version | 4.1.0 |
| Author | Askuso |

# Contents

1. Overview

2. Architecture

3. Requirements and prerequisites

4. Installation and initial configuration

5. User macros

6. System monitoring

7. Channel discovery and per-channel monitoring

8. Archive disk discovery

9. Triggers

10. Graphs and dashboard

11. Polling intervals and performance

12. Troubleshooting

13. Security considerations

14. Known limitations

15. Validation checklist

16. Change log

# 1. Overview

Template App TRASSIR Windows HTTP SDK is a Zabbix template for TRASSIR Server installations running on Windows. It communicates directly with the built-in HTTP SDK and does not require a Zabbix agent, SNMP, or external scripts.

## 1.1 Main capabilities

System health, CPU, uptime, database, storage, network and automation status.

Automatic discovery of local TRASSIR channels and archive disks.

Independent per-channel polling for signal, FPS, bitrate, recording and archive statistics.

Current archive depth, complete days, rolling seven-day average and relative archive health.

Triggers, graph prototypes and a ready-to-use overview dashboard.

# 2. Architecture

The template uses two system-level SCRIPT master items and one per-channel SCRIPT master prototype. Dependent items extract values from the returned JSON. This keeps HTTP request count controlled while isolating failures of individual cameras.

System: RAW - health and local channels

Storage: RAW - archive disks

Camera: {#CHANNEL.NAME}: RAW

# 3. Requirements and prerequisites

| Requirement | Details |
| --- | --- |
| Zabbix | 7.2 or newer for this export. |
| TRASSIR | TRASSIR Server running on Windows with HTTP SDK enabled. |
| Network | TCP access from Zabbix Server or Proxy to the SDK port. |
| Credentials | SDK password with read access. |
| Host interface | Address must resolve to the TRASSIR server. |

# 4. Installation and initial configuration

Enable the HTTP/Web SDK in TRASSIR.

Create or select a technical monitoring account and note the SDK password.

Verify TCP connectivity from the selected Zabbix Server or Proxy.

Import templates/7.2/template_app_trassir_windows_http_sdk.xml.

Create a host and set its interface address to the TRASSIR server.

Link Template App TRASSIR Windows HTTP SDK.

Set the SDK password and adjust scheme and port.

Wait for the master items and LLD rules, or execute them manually.

# 5. User macros

| Macro | Default | Purpose |
| --- | --- | --- |
| {$TRASSIR.ARCHIVE.MIN.DAYS} | 7 | Minimum overall archive retention, in days. |
| {$TRASSIR.CHANNEL.ARCHIVE.MIN.DAYS} | 7 | Minimum per-camera archive retention, in days. |
| {$TRASSIR.CHANNEL.POLL.INTERVAL} | 5m | Per-camera polling interval. Recommended value: 5 minutes. |
| {$TRASSIR.CPU.WARN} | 85 | CPU warning threshold, percent. |
| {$TRASSIR.DISK.POLL.INTERVAL} | 5m | Archive disk polling interval. |
| {$TRASSIR.MEM.WARN} | 90 | Reserved memory warning threshold, percent. |
| {$TRASSIR.POLL.INTERVAL} | 2m | Polling interval for /health and /objects. |
| {$TRASSIR.PORT} | 8080 | TRASSIR Web SDK port. |
| {$TRASSIR.RECORD.DEBT.WARN} | 5 | Recording debt threshold, seconds. |
| {$TRASSIR.SCHEME} | https | TRASSIR Web SDK scheme: https or http. |
| {$TRASSIR.SDK.PASSWORD} |  | SDK password for the Script technical account. |
| {$TRASSIR.DISK.EXCLUDE} |  | Comma-separated list of disks whose trigger problems must be suppressed, for example C,D,R. Spaces and letter case are ignored. |

# 6. System monitoring

| Item | Key | Units | Description |
| --- | --- | --- | --- |
| Storage: Main archive retention | trassir.archive.main.days | d | TRASSIR monitoring metric. See the repository documentation for details. |
| Storage: Private archive retention | trassir.archive.private.days | d | TRASSIR monitoring metric. See the repository documentation for details. |
| Storage: Substream archive retention | trassir.archive.sub.days | d | TRASSIR monitoring metric. See the repository documentation for details. |
| System: Automation status | trassir.automation.status |  | TRASSIR monitoring metric. See the repository documentation for details. |
| Summary: Local camera availability | trassir.channels.availability.pct | % | TRASSIR monitoring metric. See the repository documentation for details. |
| Summary: Offline local cameras | trassir.channels.local.offline |  | TRASSIR monitoring metric. See the repository documentation for details. |
| Summary: Online local cameras | trassir.channels.local.online |  | TRASSIR monitoring metric. See the repository documentation for details. |
| Summary: Total local cameras | trassir.channels.local.total |  | TRASSIR monitoring metric. See the repository documentation for details. |
| Summary: Camera availability text | trassir.channels.summary |  | TRASSIR monitoring metric. See the repository documentation for details. |
| System: CPU load | trassir.cpu.usage | % | TRASSIR monitoring metric. See the repository documentation for details. |
| System: Database status | trassir.database.status |  | TRASSIR monitoring metric. See the repository documentation for details. |
| Storage: Disk status | trassir.disks.status |  | TRASSIR monitoring metric. See the repository documentation for details. |
| System: Network status | trassir.network.status |  | TRASSIR monitoring metric. See the repository documentation for details. |
| System: Uptime | trassir.uptime | uptime | TRASSIR monitoring metric. See the repository documentation for details. |

# 7. Channel discovery and per-channel monitoring

The discovery rule uses the channel GUID as a stable identifier and the camera name for visible names and tags. Each camera has its own SCRIPT master item.

| Prototype | Key | Units | Description |
| --- | --- | --- | --- |
| Camera: {#CHANNEL.NAME}: estimated daily archive usage | trassir.channel.archive.forecast.gb.day["{#CHANNEL.GUID}"] | GB | Discovered TRASSIR metric. See the repository documentation for details. |
| Camera: {#CHANNEL.NAME}: daily archive usage | trassir.channel.archive.gb.day["{#CHANNEL.GUID}"] | GB | Discovered TRASSIR metric. See the repository documentation for details. |
| Camera: {#CHANNEL.NAME}: main archive retention | trassir.channel.archive.main.days["{#CHANNEL.GUID}"] | d | Discovered TRASSIR metric. See the repository documentation for details. |
| Camera: {#CHANNEL.NAME}: substream archive retention | trassir.channel.archive.sub.days["{#CHANNEL.GUID}"] | d | Discovered TRASSIR metric. See the repository documentation for details. |
| Camera: {#CHANNEL.NAME}: bitrate exceeded | trassir.channel.bitrate.exceeded["{#CHANNEL.GUID}"] |  | Discovered TRASSIR metric. See the repository documentation for details. |
| Camera: {#CHANNEL.NAME}: main stream FPS | trassir.channel.fps.main["{#CHANNEL.GUID}"] | fps | Discovered TRASSIR metric. See the repository documentation for details. |
| Camera: {#CHANNEL.NAME}: substream FPS | trassir.channel.fps.sub["{#CHANNEL.GUID}"] | fps | Discovered TRASSIR metric. See the repository documentation for details. |
| Camera: {#CHANNEL.NAME}: capture error | trassir.channel.grabber.error["{#CHANNEL.GUID}"] |  | Discovered TRASSIR metric. See the repository documentation for details. |
| Camera: {#CHANNEL.NAME}: main stream bitrate | trassir.channel.kbps.main["{#CHANNEL.GUID}"] | Kbps | Discovered TRASSIR metric. See the repository documentation for details. |
| Camera: {#CHANNEL.NAME}: recording bitrate | trassir.channel.kbps.record["{#CHANNEL.GUID}"] | Kbps | Discovered TRASSIR metric. See the repository documentation for details. |
| Camera: {#CHANNEL.NAME}: substream bitrate | trassir.channel.kbps.sub["{#CHANNEL.GUID}"] | Kbps | Discovered TRASSIR metric. See the repository documentation for details. |
| Camera: {#CHANNEL.NAME}: slow archive merge | trassir.channel.merge.slow["{#CHANNEL.GUID}"] |  | Discovered TRASSIR metric. See the repository documentation for details. |
| Camera: {#CHANNEL.NAME}: RAW | trassir.channel.raw["{#CHANNEL.GUID}"] |  | Discovered TRASSIR metric. See the repository documentation for details. |
| Camera: {#CHANNEL.NAME}: recording debt | trassir.channel.record.debt["{#CHANNEL.GUID}"] | s | Discovered TRASSIR metric. See the repository documentation for details. |
| Camera: {#CHANNEL.NAME}: last recording error | trassir.channel.record.last_error["{#CHANNEL.GUID}"] |  | Discovered TRASSIR metric. See the repository documentation for details. |
| Camera: {#CHANNEL.NAME}: hardware recording | trassir.channel.recording.hw["{#CHANNEL.GUID}"] |  | Hardware recording state: 1 - active; 0 - inactive; -1 - not supported. A value of -1 is not an error. |
| Camera: {#CHANNEL.NAME}: local recording | trassir.channel.recording.local["{#CHANNEL.GUID}"] |  | Discovered TRASSIR metric. See the repository documentation for details. |
| Camera: {#CHANNEL.NAME}: video signal | trassir.channel.signal["{#CHANNEL.GUID}"] |  | Discovered TRASSIR metric. See the repository documentation for details. |
| Camera: {#CHANNEL.NAME}: online status | trassir.channel.status["{#CHANNEL.GUID}"] |  | Direct camera online state returned by TRASSIR. 1 - online; 0 - offline; -1 - no data. |

# 8. Archive disk discovery

Disk trigger problems can be suppressed with {$TRASSIR.DISK.EXCLUDE}. Metrics and graphs remain available.

| Prototype | Key | Units | Description |
| --- | --- | --- | --- |
| Storage: Disk {#DISK_ID}: capacity | trassir.archive.disk.capacity["{#ID}"] | GB | Discovered TRASSIR metric. See the repository documentation for details. |
| Storage: Disk {#DISK_ID}: archive retention | trassir.archive.disk.days["{#ID}"] | d | Discovered TRASSIR metric. See the repository documentation for details. |
| Storage: Disk {#DISK_ID}: enabled | trassir.archive.disk.enabled["{#ID}"] |  | Discovered TRASSIR metric. See the repository documentation for details. |
| Storage: Disk {#DISK_ID}: error counter | trassir.archive.disk.errors["{#ID}"] |  | Discovered TRASSIR metric. See the repository documentation for details. |
| Storage: Disk {#DISK_ID}: formatting required | trassir.archive.disk.format.needed["{#ID}"] |  | Discovered TRASSIR metric. See the repository documentation for details. |
| Storage: Disk {#DISK_ID}: identifier | trassir.archive.disk.id["{#ID}"] |  | Discovered TRASSIR metric. See the repository documentation for details. |
| Storage: Disk {#DISK_ID}: last error | trassir.archive.disk.last.error["{#ID}"] |  | Discovered TRASSIR metric. See the repository documentation for details. |
| Storage: Disk {#DISK_ID}: model | trassir.archive.disk.model["{#ID}"] |  | Discovered TRASSIR metric. See the repository documentation for details. |
| Storage: Disk {#DISK_ID}: read-only | trassir.archive.disk.readonly["{#ID}"] |  | Discovered TRASSIR metric. See the repository documentation for details. |
| Storage: Disk {#DISK_ID}: read speed | trassir.archive.disk.read["{#ID}"] | MBps | Discovered TRASSIR metric. See the repository documentation for details. |
| Storage: Disk {#DISK_ID}: serial number | trassir.archive.disk.serial["{#ID}"] |  | Discovered TRASSIR metric. See the repository documentation for details. |
| Storage: Disk {#DISK_ID}: current speed | trassir.archive.disk.speed["{#ID}"] | MBps | Discovered TRASSIR metric. See the repository documentation for details. |
| Storage: Disk {#DISK_ID}: unavailable | trassir.archive.disk.unavailable["{#ID}"] |  | Discovered TRASSIR metric. See the repository documentation for details. |
| Storage: Disk {#DISK_ID}: unusable | trassir.archive.disk.unusable["{#ID}"] |  | Discovered TRASSIR metric. See the repository documentation for details. |
| Storage: Disk {#DISK_ID}: write speed | trassir.archive.disk.write["{#ID}"] | MBps | Discovered TRASSIR metric. See the repository documentation for details. |

# 9. Triggers

| Trigger | Severity | Expression | Description |
| --- | --- | --- | --- |
| TRASSIR: Main archive retention below {$TRASSIR.ARCHIVE.MIN.DAYS} days | AVERAGE | last(/Template App TRASSIR Windows HTTP SDK/trassir.archive.main.days)<{$TRASSIR.ARCHIVE.MIN.DAYS} | TRASSIR monitoring trigger. See the repository documentation for details. |
| TRASSIR: Offline local cameras detected | HIGH | last(/Template App TRASSIR Windows HTTP SDK/trassir.channels.local.offline)>0 | TRASSIR monitoring trigger. See the repository documentation for details. |
| TRASSIR: High CPU load | WARNING | avg(/Template App TRASSIR Windows HTTP SDK/trassir.cpu.usage,10m)>{$TRASSIR.CPU.WARN} | TRASSIR monitoring trigger. See the repository documentation for details. |
| TRASSIR: Database status error | HIGH | last(/Template App TRASSIR Windows HTTP SDK/trassir.database.status)=0 | TRASSIR monitoring trigger. See the repository documentation for details. |
| TRASSIR: Disk status error | HIGH | last(/Template App TRASSIR Windows HTTP SDK/trassir.disks.status)=0 | TRASSIR monitoring trigger. See the repository documentation for details. |
| TRASSIR: Network status error | AVERAGE | last(/Template App TRASSIR Windows HTTP SDK/trassir.network.status)=0 | TRASSIR monitoring trigger. See the repository documentation for details. |
| TRASSIR: No aggregate data from SDK | HIGH | nodata(/Template App TRASSIR Windows HTTP SDK/trassir.raw.aggregate,5m)=1 | TRASSIR monitoring trigger. See the repository documentation for details. |
| TRASSIR: SDK aggregate polling errors | WARNING | length(last(/Template App TRASSIR Windows HTTP SDK/trassir.raw.errors))>2 | TRASSIR monitoring trigger. See the repository documentation for details. |
| Storage: Archive retention below 90% of the 7-day average | WARNING | max(/Template App TRASSIR Windows HTTP SDK/trassir.archive.main.baseline.pct,30m)<90 | TRASSIR monitoring trigger. See the repository documentation for details. |
| Storage: Archive retention below 80% of the 7-day average | HIGH | max(/Template App TRASSIR Windows HTTP SDK/trassir.archive.main.baseline.pct,30m)<80 | TRASSIR monitoring trigger. See the repository documentation for details. |
| TRASSIR: Archive disk {#DISK_ID} is disabled | HIGH | ((max(/Template App TRASSIR Windows HTTP SDK/trassir.archive.disk.enabled["{#ID}"],5m)=0)) and {#DISK.MONITOR}=1 | TRASSIR monitoring trigger. See the repository documentation for details. |
| TRASSIR: Archive disk {#DISK_ID} error counter increased | HIGH | ((change(/Template App TRASSIR Windows HTTP SDK/trassir.archive.disk.errors["{#ID}"])>0)) and {#DISK.MONITOR}=1 | TRASSIR monitoring trigger. See the repository documentation for details. |
| TRASSIR: Archive disk {#DISK_ID} requires formatting | HIGH | ((max(/Template App TRASSIR Windows HTTP SDK/trassir.archive.disk.format.needed["{#ID}"],5m)=1)) and {#DISK.MONITOR}=1 | TRASSIR monitoring trigger. See the repository documentation for details. |
| TRASSIR: Archive disk {#DISK_ID} switched to read-only mode | HIGH | ((max(/Template App TRASSIR Windows HTTP SDK/trassir.archive.disk.readonly["{#ID}"],5m)=1)) and {#DISK.MONITOR}=1 | TRASSIR monitoring trigger. See the repository documentation for details. |
| TRASSIR: Archive disk {#DISK_ID} is unavailable | DISASTER | ((max(/Template App TRASSIR Windows HTTP SDK/trassir.archive.disk.unavailable["{#ID}"],5m)=1)) and {#DISK.MONITOR}=1 | TRASSIR monitoring trigger. See the repository documentation for details. |
| TRASSIR: Archive disk {#DISK_ID} is unusable | DISASTER | ((max(/Template App TRASSIR Windows HTTP SDK/trassir.archive.disk.unusable["{#ID}"],5m)=1)) and {#DISK.MONITOR}=1 | TRASSIR monitoring trigger. See the repository documentation for details. |
| Camera: "{#CHANNEL.NAME}" archive retention is below {$TRASSIR.CHANNEL.ARCHIVE.MIN.DAYS} days | WARNING | max(/Template App TRASSIR Windows HTTP SDK/trassir.channel.archive.main.days["{#CHANNEL.GUID}"],6h)<{$TRASSIR.CHANNEL.ARCHIVE.MIN.DAYS} and max(/Template App TRASSIR Windows HTTP SDK/trassir.channel.archive.main.days["{#CHANNEL.GUID}"],6h)>=0 | TRASSIR monitoring trigger. See the repository documentation for details. |
| Camera: "{#CHANNEL.NAME}" exceeded the allowed bitrate | WARNING | max(/Template App TRASSIR Windows HTTP SDK/trassir.channel.bitrate.exceeded["{#CHANNEL.GUID}"],5m)=1 | TRASSIR monitoring trigger. See the repository documentation for details. |
| Camera: "{#CHANNEL.NAME}" archive merging is too slow | WARNING | max(/Template App TRASSIR Windows HTTP SDK/trassir.channel.merge.slow["{#CHANNEL.GUID}"],5m)=1 | TRASSIR monitoring trigger. See the repository documentation for details. |
| Camera: "{#CHANNEL.NAME}" recording delay is increasing | AVERAGE | min(/Template App TRASSIR Windows HTTP SDK/trassir.channel.record.debt["{#CHANNEL.GUID}"],5m)>{$TRASSIR.RECORD.DEBT.WARN} | TRASSIR monitoring trigger. See the repository documentation for details. |
| Camera: "{#CHANNEL.NAME}" reports a recording error | WARNING | length(last(/Template App TRASSIR Windows HTTP SDK/trassir.channel.record.last_error["{#CHANNEL.GUID}"]))>0 | TRASSIR monitoring trigger. See the repository documentation for details. |
| Camera: "{#CHANNEL.NAME}" lost video signal | HIGH | max(/Template App TRASSIR Windows HTTP SDK/trassir.channel.signal["{#CHANNEL.GUID}"],3m)=0 | TRASSIR monitoring trigger. See the repository documentation for details. |
| Camera: "{#CHANNEL.NAME}" is offline | DISASTER | max(/Template App TRASSIR Windows HTTP SDK/trassir.channel.status["{#CHANNEL.GUID}"],3m)=0 | TRASSIR monitoring trigger. See the repository documentation for details. |

# 10. Graphs and dashboard

CPU history and gauge.

Camera availability summary and gauge.

Main archive depth, complete days and seven-day average.

Archive-relative-to-baseline gauge.

Per-camera stream and archive graphs.

Per-disk performance and archive graphs.

Active TRASSIR problems.

# 11. Polling intervals and performance

Health and object tree: 2 minutes.

Every camera: 5 minutes.

Archive disks: 5 minutes.

Archive calculated items: 10 minutes.

# 12. Troubleshooting

Check the system RAW item and aggregate error item.

Check per-camera RAW items.

Verify host address, scheme, port and password.

Verify the assigned proxy and firewall.

Execute LLD rules after changing disk exclusions.

A grey ZBX icon is normal because the template does not use Zabbix Agent.

# 13. Security considerations

Use a dedicated SDK credential.

Prefer HTTPS.

Restrict network access to the SDK port.

Do not publish passwords, session IDs, IP addresses or camera GUIDs.

# 14. Known limitations

Windows Server only; hardware NVRs require a separate template.

Local channels only.

Hardware recording may be unsupported.

RAM usage is not included.

Archive baseline requires history.

# 15. Validation checklist

RAW items have recent JSON values.

System statuses display OK.

Expected cameras and disks are discovered.

Dashboard widgets show current data.

Excluded disks do not generate trigger problems.

# 16. Change log

4.1.0 - Initial public release of the Windows-specific template.